COMPLIANCE IS MANDATORY

| Responsible Office: |  | 549 / Environmental Test Engineering and Integration Branch |
| --- | --- | --- |
| Title: | Structural Dynamics Process Management Plan |  |

Preface

- PURPOSE

This document sets forth requirements for the structural dynamic test engineering process, outlining what is required in 549-PG-8700.1.1 (Environmental Test Engineering and Integration Process) to conform to the GSFC ISO 9000 policy.

- REFERENCES

a. GSFC Document GEVS-SE, “ General Environmental Verification Specification for STS and ELV Payloads, Systems and Components”, Revision A dated June 1996.

b. Environmental Test Engineering and Integration Process, 549-PG-8700.1.1

c. MSC Safety Manual

d. SSPP-PROC-102; Bldg. 5 Shaker Operating Procedure

e. GSFC Catalog titled ‘ESD Facilities & Capabilities’ dated Jan 1995

- SCOPE

This procedure applies to the structural dynamics test engineering test process under the authority of Code 549.2

- DEFINITIONS

a. Process Owner (PO): Code 549.2 Structural Dynamic Test Engineering Section Head

b. Test Engineer (TE): The engineer designated by the PO to execute the test process.

c. Product Design Lead (PDL): The project manager or leader responsible for the test article.

d. Project Test Plan (test plan): Provided by the PDL describing the overall plan for conducting a particular test.

e. Process Operating Procedure (POP):  generated for each area of structural dynamics testing and is used as a guideline for the operational process

f. Test Procedure: Test item specific document. Derived from POP and Test Plan.

- AUTHORITIES AND RESPONSIBILITIES

N/A

- CANCELLATION

549-PG-8700.1.2 Rev A

- QUALITY RECORDS

Quality Records

Quality records will be maintained for a period of 5 years formation  is available in the attached quality record matrix. All civil service generated quality records will be maintained by the Section Head and listed in the Quality Record Master list under “Structural Dynamics”.

- Quality Record Matrix Definitions:

- STTS: Static Testing Technical Support

- DT: Dynamic Testing

- EDS: Engineering Design and Support

- RA: Repository Administrator

- DR: Document Repository

- TE: Test Engineer

G:  TC: Test Conductor

| ###### Process | Quality Record Title/Number | Record Custodian | File Location | Retention Period |
| --- | --- | --- | --- | --- |
| 09PC-PP01-2 | Steady State Acceleration Testing |  |  |  |
| 09PC-PP01-2 | Test Procedure | Sandy Broadwater | Bldg. 7, Rm 148 | 5 years |
| 09PC-PP01-2 | Test Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 years |
| 09PC-PP01-2 | Test Data | Included in Test Report | N/A | 5 years |
| 09PC-PP01-2 | Maintenance Records | Phil Matthews | Bldg. 15, Room 104A | 5 years |
| 09PC-PP01-2 | Calibration Records | Phil Matthews | Bldg. 29, High Bay | 5 years |
| 09PC-PP01-2 | Stress Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 years |
| 09PC-PP01-2 | Fixture Drawings | Mark Parker/Ron Cary | Bldg. 7, Cage 15A | 5 years |
| ##### 09PC-PP02-2 | Vibration Testing |  |  |  |
| 09PC-PP02-2 | "(1) Test Folio consisting of test procedure, report, and data | Don Baker / Mark Ramsey | Bldg. 7, Rm 030 | 5 Years |
| 09PC-PP02-2 | Maintenance Records | Mark Ramsey / Jack Bauernschmidt | Bldg. 7, A Mezzanine | 5 Years |
| 09PC-PP02-2 | Accelerometer Calibration Records | Mark Ramsey / Bob Denhardt | Bldg. 7, Rm 41 | 5 Years |
| 09PC-WI36-1 | Vibration Test Facility Operating Procedure |  |  |  |
| 09PC-WI36-1 | Data Acquisition Folio | Don Baker | Bldg. 7, Rm 024 | 5 Years |
| 09PC-WI36-1 | Vibration Test Report | Don Baker | Bldg. 7, Rm 030 | 5 Years |
| 09PC-WI36-1 | Test Folio | Don Baker | Bldg. 7, Rm 030 | 5 Years |
| 09PC-WI36-1 | Vibration Control System Processed Data & Operational Parameter File | Chris McLeod | Bldg. 7, Rm 018 | 5 Years |
| 09PC-WI36-1 | DDAS | Chris McLeod | Bldg. 7, Rm 018 | 5 Years |
| 09PC-WI36-1 | Vib. Facility Maintenance File | Jack Bauernschmidt | Bldg. 7, Rm 024 | 5 Years |
| 09PC-WI37-0 | Vibration Test Facility Maintenance Plan |  |  |  |
| 09PC-WI37-0 | Vibration System Weekly Operational Readiness Inspection | Jack Bauernschmidt | Bldg. 7, Rm M-1 | 5 Years |
| 09PC-WI37-0 | Function 23 Maintenance Record | Jack Bauernschmidt | Bldg. 7, Rm M-1 | 5 Years |
| 09PC-PP03-2 | Modal Survey Testing |  |  |  |
| 09-PC-PP03-2 | Test Procedure | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09-PC-PP03-2 | Test Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09-PC-PP03-2 | Test Data | Mark Ramsey | Optical Disc, Bldg. 7, Rm 024 | 5 Years |
| 09-PC-PP03-2 | Test Folio | Mark Ramsey | Bldg. 15, Modal Trailer | 5 Years |
| 09-PC-PP03-2 | Test Logbook | Mark Ramsey | Bldg 15, Modal Trailer | 5 Years |
| 09-PC-PP03-2 | Facility Modification Records | Mark Ramsey | Bldg. 15, Modal Trailer | 5 Years |
| 09-PC-PP03-2 | Accelerometer Calibration Records | Mark Ramsey | Vibration Lab | 5 Years |
| 09-PC-PP03-2 | Fixture Drawings | Mark Parker / Ron Cary | Bldg. 7, Cage 15A | 5 Years |
| 09-PC-PP03-2 | Stress Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP04-2 | Mass Properties Measurement System |  |  |  |
| 09PC-PP04-2 | Test Procedure | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP04-2 | Test Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP04-2 | Test Data | N/A | Included in Test Report | 5 Years |
| 09PC-PP04-2 | Maintenance Records | Phil Matthews | Bldg. 15 High-Bay File Cabinet | 5 Years |
| 09PC-PP04-2 | Facility Modification Records | Phil Matthews | Bldg. 15 High-Bay File Cabinet | 5 Years |
| 09PC-PP04-2 | Stress Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP04-2 | Fixture Drawings | Mark Parker / Ron Cary | Bldg. 7, Cage 15A | 5 Years |
| ##### 09PC-PP05-3 | Static Load Testing |  |  |  |
| 09PC-PP05-3 | Test Procedure | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP05-3 | Test Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP05-3 | Test Data | N/A | Included in Test Report | 5 Years |
| 09PC-PP05-3 | Maintenance Records | Phil Matthews | Bldg. 15, Rm 104 | 5 Years |
| 09PC-PP05-3 | Facility Modification Records | Phil Matthews | Bldg. 15, Rm 104 | 5 Years |
| 09PC-PP05-3 | Calibration Records | Phil Matthews | Bldg. 29, High Bay | 5 Years |
| 09PC-PP05-3 | Stress Report | Sandy Broadwater | Bldg. 7, Rm 148 | 5 Years |
| 09PC-PP05-3 | Fixture Drawings | Mark Parker / Ron Cary | Bldg. 7, Cage 15A | 5 Years |
| 09PC-WI42-0 | Load Cell Calibration Verification Procedure |  |  |  |
| 09PC-WI42-0 | Load Cell Calibration Verification Data | Phil Matthews | Load Cell File Cabinet CC: Cabinet load cells are stored | 5 Years |
| 09PC-PP06-0 | Acoustic Testing |  |  |  |
| 09PC-PP06-0 | Test Procedure | Sandy Broadwater | Test Folio | 5 Years |
| 09PC-PP06-0 | Test Report | Sandy Broadwater | Bldg. 7, Rm 178 | 5 Years |
| 09PC-PP06-0 | Test Data | N /A | Included in Test Report | 5 Years |
| 09PC-PP06-0 | Test Folio | KC Shah / Phil Matthews | Bldg. 10, Rm 104 | 5 Years |
| 09PC-PP06-0 | Maintenance Records | KC Shah / Phil Matthews | Bldg. 10, Rm 104 | 5 Years |
| 09PC-PP06-0 | Facility Modification Records | KC Shah / Phil Matthews | Bldg. 10, Rm 104 | 5 Years |
| 09PC-PP06-0 | Calibration Records | KC Shah / Phil Matthews | Bldg. 10, Rm 104 | 5 Years |
| 11CI-WI02-2 | Steady State Accelerometer Calibration |  |  |  |
| 11CI-WI02-2 | Calibration Data Sheet | Phil Matthews | Bldg. 7, Cage 15A | 3 years or Contract Life |
| Structural Dynamics |  |  |  |  |
| Structural Dynamics | Test Report | C. Mattiello | Bldg. 7 Room 165 | 5 Years |
| Structural Dynamics | Problem Review | C. Mattiello | Bldg. 7 Room 165 | Indefinitely |
| Structural Dynamics | Equipment Repair/Modification Completion | C. Mattiello | Bldg. 7 Room 165 | Indefinitely |
| Structural Dynamics | Readiness Review | C. Mattiello | Bldg. 7 Room 165 | Indefinitely |
| Structural Dynamics | Post Test Review | C. Mattiello | Bldg. 7 Room 165 | Indefinitely |

Implementation

In this document, a requirement is identified by “shall,” a good practice by “should,” permission by “may” or “can,” expectation by “will,” and descriptive material by “is.”

1.1	Process Description

The structural dynamics testing process can include both tests and structural characterizations used to meet the established criteria set forth in the test plan. These tests are as follows:

- Vibration Testing

Modal Survey Testing

Mass Properties Testing

Static Loads Testing

Acoustic Testing

Steady State Acceleration

1.2	Requirements

Specific product test requirements and conditions are identified in the test plan. The POP provides the guidelines for the operational process. A specific test procedure is generated from a POP with the addition of product specific test requirements and test conditions.

1.3	 Safety

Safety to personnel and hardware are given top priority. All operations will adhere to the safety requirements outlined in the MSC Safety Manual.

1.4	Cost/Schedule

Cost estimates are provided to the PDL or his designee based on the best available information. These costs will be provided prior to testing. If the estimate/scope changes by more than 15%, the test costs will be adjusted and a justification submitted to the PDL or his designee.  So that testing can proceed quickly, the PDL will be made aware verbally of the cost increases. A written re-estimate will be made at the earliest convenience. Because of the dynamics involved, for vibration testing initially estimated at $5K or less, a re-estimate will be submitted to the PDL as the test engineer deems necessary. The PO keeps the overall test schedule. Test Priorities are set at the branch level in accordance with the Center priorities.

1.5	Procedure

Each test process has a POP as follows:

Test Name 							POP No.

Vibration Testing						09PC-PP02

Modal Survey Testing						09PC-PP03

Mass Properties Testing					09PC-PP04

Static Loads Testing						09PC-PP05

Acoustics Test							09PC-PP06

Steady State Acceleration Testing				09PC-PP01

  - Equipment

Structural dynamic test facilities and equipment are described in the ESD Facilities and Capabilities Manual. The manual provides physical descriptions and performance capabilities for test equipment. The capacities and performance capabilities of the equipment are suitable for conducting tests as specified in GEVS-SE. The facility includes cranes and handling equipment appropriate for staging space flight hardware. The facility provides a suitable work environment for safely and efficiently conducting structural and dynamic tests. In addition, there is a vibration shaker located in bldg. 5. It is considered as an R&D shaker and not subject to the same quality control measures as the shakers in the basement of building 7. There is a procedure used for operation; SSPP-PROC-102. It is on hand at the facility.  Basically, the user is responsible for their own quality control. There is also a 6 degree-of-freedom vibration shaker located in the main vibration facility. This shaker system is under development and is not considered a resource for testing at this time.

1.7	Process Control

All structural testing is monitored by various methods to ensure adherence to agreed upon test requirements. See the POP's for details. Operational readiness reviews will be scheduled at the discretion of the PO. These reviews are held prior to complex tests to ensure that all technical requirements are being met and that all safety, proof testing and handling issues are resolved.

1.8	Process Approval

All POP's will be approved by the PO. All test procedures will be signed-off by the TE and PDL as a minimum prior to testing. If there are changes, the approved test procedure will be “red lined” and initialed and dated by the TE and PDL. In some cases, engineering or R&D type testing will be done and will not require a formal test plan from the PDL or test procedure from the TE. This type of testing will only be performed on non-flight hardware and allows for maximum flexibility in the "real time" adjustment of specifications and conduct of test.

1.9	Criteria for Workmanship

Prior to each test, facility checkouts are done to ensure readiness.  Also, control parameters are monitored during testing and post test data is reviewed to ensure compliance with agreed upon specifications. See the POP's for details.

1.10	Corrective Actions

The 549.2 response to situations that happen during the testing process, are covered in the POP’s.  In addition to the corrective actions taken, incident reports are written for those items that meet the incident report criteria in the MSC Safety Manual.

1.11	Continuous Improvement

Upon completion of a major test, a post test review will be held to discuss the conduct of the testing process, recommend changes and generate a lessons learned list. These post test reviews will be held at the PO’s discretion.

1.12	Equipment Maintenance

Maintenance for required facilities are defined in the POPs. As a general rule, a tolerance of one month is allowed for scheduled maintenance completion, but ultimately the workload determines when maintenance can be performed. Equipment deemed unacceptable for use will be removed from service.

1.13	Calibration

Transducer calibrations are done as scheduled. Instrumentation, gages, etc. not on a calibration cycle are marked as such. Calibration specifics are covered in or referenced in the POP’s.

1.14	Equipment Repair/Modifications

When a significant problem with test process equipment arises, a problem report is generated and submitted to the PO who will keep it on file.  The TE will determine whether or not a problem is significant. When repair or modifications to equipment essential to the test process has been verbally approved by the PO, a memo addressing the completion of said work will be sent to the PO from the contractor. Also, modifications to facilities can be made with PO approval on the basis of improvement or project requirement.

CHANGE HISTORY LOG

| Revision | Effective Date | Description of Changes |
| --- | --- | --- |
| Baseline | 02/07/1999 | Initial Release |
| A | 03/09/2000 | Updated quality record list. |
| B | 03/06/2001 | Changes to paragraphs 1.4 and 1.5 Quality records updated. Updated format to current version. |