Process Specification for Radiographic Inspection

Engineering Directorate Structural Engineering Division February 2007 National Aeronautics and Space Administration Lyndon B. Johnson Space Center Houston, Texas

Process Specification for Radiographic Inspection

| Prepared by: |  |  |  |
| --- | --- | --- | --- |
|  | Ajay M. Koshti, ES4 |  | Date |
|  | Norman P. Ruffino, ESCG-ES4 |  | Date |

| Approved by: |  |  |  |
| --- | --- | --- | --- |
|  | Bradley S. Files Chief, Materials and Processes Branch/ES4 |  | Date |

| REVISIONS |  |  |
| --- | --- | --- |
| VERSION | CHANGES | DATE |
| Baseline | Original version | 7/26/99 |
| A | Reviewed document per QMS requirements. Updated division name, organization codes, and document numbers. | 12/19/02 |
| B | Deleted SNT-TC-1A and MSFC-STD-1249. Added NASA-STD-5009 and NASA-STD-5019. Added definition of discontinuity and indication. | 2/14/07 |

1.0	SCOPE

This process specification establishes the minimum requirements for radiographic inspection of metallic and nonmetallic materials, components, and assemblies.

# 2.0	APPLICABILITY

This specification is applicable to in-process, final, and in-service radiographic inspections to detect cracks, porosity, voids, inclusions, improper assembly, density variations, corrosion, and other discontinuities.

3.0	USAGE

This specification shall be invoked by including an inspection note on the applicable engineering drawing or by reference in a Process Specification, Task Performance Sheet, Discrepancy Report/Material Review Record, or other appropriate document.  The engineering drawing or referencing document shall specify the criteria by which components are judged acceptable.  An example of acceptance criteria for welded components are contained in NAS 1514.  When there are different acceptance criteria for different areas on a component, the drawing shall be zoned with the acceptance criteria identified for each zone.  If the number of components to be inspected and the amount of coverage of each component are not specified, all components shall be examined and shall receive 100 percent radiographic coverage.

A typical radiographic inspection note for welded flight hardware is given in Figure 1.

PERFORM RADIOGRAPHIC INSPECTION OF WELDS PER JSC PRC-6503, ACCEPTANCE CRITERIA PER NAS 1514, CLASS I.

Figure 1.

3.1	INSPECTION SEQUENCE

The stage in the manufacturing process where radiographic inspection is performed should be specified on the engineering drawing or in the referencing document.  Factors to be considered when specifying the inspection sequence are: the accessibility of both sides of the component; the complexity of the assembly as it relates to optimal alignment of the radiation beam; loading of the component or assembly (compressive loads reduce cracks detectability); and the need for subsequent manufacturing processes that could generate discontinuities.  Radiographic inspection may be performed before heat treatment, provided liquid penetrant or magnetic particle inspections are performed after heat treatment.

3.2	SPECIAL NDE OF FRACTURE CRITICAL COMPONENTS

When implementation of fracture control requirements necessitates Special Nondestructive Evaluation (NDE) of a fracture critical component, the requirement for Special NDE shall appear in the inspection note as shown in Figure 2.   When Special NDE is required, the specific inspection procedure and inspector shall be qualified in accordance with Section 7.0.

Figure 2.

PERFORM RADIOGRAPHIC INSPECTION OF WELDS PER JSC PRC-6503, ACCEPTANCE CRITERIA PER NAS 1514, CLASS I.  SPECIAL NDE QUALIFICATION REQUIRED.

4.0	REFERENCES

The following specifications, standards, and handbooks form a part of this document to the extent specified herein.  All documents listed are assumed to be the current revision unless a specific revision is listed.  In the case of conflict between this specification and the technical requirements cited in other referenced documents, the requirements of this document take precedence.

ASTM E 1742		Standard Practice for Radiographic Examination

NASA-STD-5009	Nondestructive Evaluation Requirements for Fracture Critical Metallic Components

NAS 410				NAS Certification & Qualification of Nondestructive

Test Personnel

NAS 1514	Radiographic Standard for Classification of Fusion Weld Discontinuities

NASA-STD-5003	Fracture Control Requirements for Payloads Using the Space Shuttle

# NASA-STD-5019	fracture Control Requirements for Spaceflight Hardware

SSP 30558	Fracture Control Requirements for Space Station

The following references were used to develop this process specification:

SOP-007.1	Preparation and Revision of Process Specifications (PRC’s)

JSC 8500	Engineering Drawing System Requirements

5.0	MATERIAL REQUIREMENTS

Film, non-film recording media, and film processing solutions shall be in accordance with ASTM E 1742.

6.0	PROCESS REQUIREMENTS

6.1	GENERAL

Radiographic inspections shall be performed in accordance with ASTM E 1742 except as modified by this specification.

6.2	WRITTEN PROCEDURES

A detailed written procedure shall be prepared for each part to be inspected.  The procedure shall meet the requirements of this specification and shall ensure the consistency and reproducibility of the inspection at the required radiographic quality level.  General procedures covering a variety of different parts may be used provided they meet the requirements of this procedure and clearly apply to the parts to be inspected.  When general procedures are used, a written part specific technique shall be prepared.  At a minimum, the part specific procedure or the general procedure and part specific technique shall cover all of the information required by ASTM E 1742.

For work performed at JSC facilities, written procedures should consist of Detailed Process Instructions (DPIs) selected for use from the DPI-6503-XX series of work instructions.

6.3	FILM DENSITY

Film density in the area of interest shall be in the range from 2.0 to 4.0 (inclusive).  Film densities greater than 4.0 are permitted when approved by the JSC Materials and Processes Branch (ES4).  Densities less than 2.0 are permitted only when items not requiring an image quality indicator (IQI) are inspected.

6.4	APPLICABLE AREA

For components requiring an IQI, only the area of the film that falls within a 10 degree cone of radiation (10 degree total solid angle, apex at the radiation source, central axis of the cone equal to the central axis of the radiation beam) shall be considered valid for interpretation.

6.5	SOURCE-TO-FILM DISTANCE

The minimum allowable source-to-film distance for flight hardware shall be calculated using a maximum geometric unsharpness (Ug) of 0.003 inch.

6.6	FRACTURE CRITICAL COMPONENTS

Radiographic inspections of fracture critical components shall be performed in accordance with the process requirements in NASA-STD-5009.  The requirements in NASA-STD-5009 not otherwise covered in this specification are included in the following:

a.	The Radiographic Quality Level shall be at least Level 1 (2 - 1T);

b.	Film density in the area of interest shall be in the range from 3.0 to 4.0 (inclusive);

c.	Only the area of the film that falls within a 5 degree cone of radiation (5 degree total solid angle, apex at the radiation source, central axis of the cone equal to the central axis of the radiation beam) shall be considered valid for interpretation;

d.	Potential crack-like flaws shall be oriented in the plane of radiation;

e.	Components shall be of consistent thickness; and

f.	Film shall be placed in close contact with the component.

When Special NDE is specified by the engineering drawing or referencing document, the inspection procedure and inspector shall be qualified in accordance with Section 7.0.

7.0	SPECIAL NDE QUALIFICATION

Use of Special NDE in accordance with NASA-STD-5019, NASA-STD-5003 or SSP 30558 requires formal demonstration of the capability to detect flaws at least as small as the critical initial crack size for the specific component to a 90/95 detection level. Each procedure, procedure application, and operator must demonstrate the required capability.  Requests for Special NDE qualification shall be directed to the JSC Materials and Processes Branch (ES4).

## 8.0	DEVIATIONS AND WAIVERS

Any deviations or waivers regarding the use of this process specification shall be requested in writing.  This request shall be directed to the JSC Materials and Processes Branch (ES4) with the appropriate justification and rationale.  A written response will be provided upon such a request.

9.0	TRAINING AND CERTIFICATION OF PERSONNEL

Personnel performing radiographic inspections shall be qualified and certified in accordance with NAS 410. Personnel making accept/reject decisions shall be certified to at least Level II. Personnel performing inspections on fracture critical components shall, at a minimum, be certified to Level II.

For work performed at JSC facilities, these requirements may be satisfied by the training and certification of personnel per the TI-6503-XX series of Training Instructions (TIs).

10.0	DEFINITIONS

90/95  	The point where the 95% lower confidence bound on the

Probability of Detection (POD) vs. flaw size curve crosses 90% POD or 90% POD with 95% lower confidence bound.

Final Inspection	The final inspection performed for the acceptance of the component.

Fracture Critical	Classification which assumes that fracture or failure of the

Component	component resulting from the occurrence of a crack will result in a catastrophic hazard.  Fracture critical components will be identified as such on the engineering drawing.

In-Process	Inspections that occur during manufacturing before a component is in final form.

In-Service	Inspections performed on components that are in use or storage.

Special NDE	A fracture control term denoting nondestructive inspection personnel, procedures, and equipment with a demonstrated capability to reliably (90/95) detect flaws smaller than those normally detected by Standard NDE procedures.

Discontinuity	An intentional or unintentional interruption in the physical structure or configuration of a material or component that may be detectable by nondestructive testing; a flaw. Discontinuities are not necessarily rejectable.

Indication	Evidence of a discontinuity that requires interpretation to determine its significance.